Wetlands

Wetlands typically are characterized by presence of hydric soils and plant species adapted to those soils. Wetlands can have high plant diversity, complex vegetative structure, and a diversity of wildlife foods, including arthropods, fruits, and nuts. Examples of wetlands in forested environments include river bottoms, stream corridors, and depressional ponds. Wetlands are especially important for amphibians, insects, and reptiles that rely on water for one or more parts of their life cycles (Batzer and Wissinger 1996; Semlitsch and Bodie 2003). Specifically, many forest amphibians rely on ephemeral pools to breed because they hold water only temporarily and thus do not harbor fish predators that typically consume their eggs (DiMauro and Hunter 2002). Waterfowl and wading birds also rely on wetlands for food and cover, often year-round (Isola et al. 2000). In drier regions, wetlands may provide important sources of drinking water for wildlife.

Given their high value to wildlife, wetland management is an important consideration during forest harvest activities. Buffers parallel to rivers and streams or surrounding small, isolated wetlands established prior to timber harvest typically are effective means by which to reduce potentially negative impacts on wetlands during timber harvest (Semlitsch and Bodie 2003). These forested buffers provide shade to maintain water temperatures and help prevent sediment deposition during timber harvest. On the other hand, buffers also may reduce the potential for disturbances emulating those that occurred naturally (e.g., fires, hurricanes). Buffer specifications vary based on target wildlife species, stream width, harvest intensity, slope, forest ecosystem, and the type of natural and anthropogenic disturbances (Russell et al. 2002, 2005). In the United States, each state has developed Best Management Practices (BMPs) for forest management that center on protecting water quality during forest harvest activities. BMPs are tailored for each states’ local conditions and, when implemented properly, are effective means by which to manage and protect forest wetlands.